SOFTWARE/ APP DEVOLPEMENT

API & Model Integration Plan

1. Speech-to-Text (S-to-T)

- Goal: Convert the raw audio file (transferred via Bluetooth) into a clean, time-stamped text transcript.

- Recommended Tool: OpenAI Whisper or Google Speech-to-Text API.

  - Whisper (Recommended for Prototype): Run a local/cloud instance of the Whisper model. It's excellent for handling diverse accents, which is crucial for the Indian classroom setting.

  - Google API: More robust for production but might require more setup for accent tuning and is cost-dependent.

- Input: The raw audio file (.wav or .mp3) transferred from the watch.

- Output: A clean, chunked text transcript (removing filler words, duplicates, irrelevant phrases) with speaker/time segmentation.

- Actionable Step for Devs: Test the chosen tool with 1-2 minutes of Indian English lecture audio to verify accent accuracy.

2. Summarization and Structuring

- Goal: Convert the raw transcript into concise, structured notes that include academic context (definitions, theorems, examples).

- Recommended Tool: Hugging Face T5 or Pegasus.

  - T5 (Text-to-Text Transfer Transformer): Highly versatile for summarization. It can be fine-tuned (or prompted heavily) to extract key points and structure the notes.

- Input: The clean, raw text transcript from the S-to-T step.

- Output: Structured Study Notes, including bullet points, headings, and clearly extracted academic terms (like definitions, theorems, and examples).

- Actionable Step for Devs: Focus on prompt engineering. The prompt for the summarization model should explicitly request a student-friendly output format (e.g., "Summarize this lecture transcript. Identify and list all definitions, theorems, and key examples in a bulleted format.").

3. Fact Verification and Confidence Checking

- Goal: Compare extracted information with trusted sources and flag any uncertain statements.

- Recommended Tools: Wikipedia API + GPT-based Cross-checks.

  - Wikipedia API: Used for fast lookups of extracted key terms (like names, dates, definitions) to confirm accuracy.

  - GPT-Based Cross-checks: A Generative AI model (like a smaller GPT model) can be used to compare specific statements from the lecture against a trusted knowledge base or a quick web search (simulating a knowledge base lookup).

- Input: The Structured Study Notes from the summarization step.

- Output: The final notes, but with:

  - A Confidence Indicator for the overall notes.

  - A Flag next to any statement that the checker could not verify or found uncertain.

- Actionable Step for Devs: Define what constitutes a "low-confidence" statement and how the flag is visually rendered in the app (e.g., red highlight or an "unverified" icon)

Hardware devolopement

Week 1 Focus: Recording and Storage (Hardware Dev 3)

| Component | Recommended Part / Action | Rationale |
| --- | --- | --- |
| Microcontroller | ESP32-S3 WROOM Module 1 | Chosen for integrated Bluetooth, low power consumption, and efficient audio handling2. |
| Microphone | MEMS I2S Microphone Module (e.g., INMP441) 3 | I2S protocol provides much cleaner, digital audio than analog mics. The MEMS design is small and less susceptible to vibration noise4. |
| Storage | MicroSD Card Module 5 | Mandatory for a 10-minute audio file. Audio files are large. The ESP32's built-in flash memory is too small for continuous lecture recording6. |

Firmware Tasks (Person 3)

- Core Setup: Program the ESP32-S3 using the Arduino IDE or ESP-IDF.

- I2S Integration: Write the code to configure the I2S peripheral pins and sample rate to read audio data from the MEMS microphone.

- SD Card File System: Implement the SPI/SD protocol to write the continuous audio stream directly to a file on the MicroSD card (e.g., a .WAV file).

- Hands-free control: Write the basic firmware for the single button/LEDs to start and stop the recording process7777.

🎯 Week 2 Focus: Bluetooth Sync (Hardware Dev 4)

The goal here is to establish the link to the mobile app and successfully move a file.

| Strategy | Focus Area | Actionable Steps (Person 4) |
| --- | --- | --- |
| Bluetooth Service | BLE GATT Server | 1. Setup GATT Server: Configure the ESP32 to act as a Bluetooth Low Energy (BLE) GATT Server. This allows it to advertise its presence (e.g., as "EduNote Watch") and define services. |
| Data Transfer | Custom File Transfer Service | 2. Define a Custom Service: Create a specific BLE service (e.g., FileTransferService) with a dedicated Characteristic (e.g., FileCharacteristic). |
|  |  | 3. Chunking Logic: Since BLE characteristics have a limited payload size (around 20-250 bytes), implement file chunking logic. Read the audio file from the SD card in small pieces and write them sequentially to the BLE Characteristic. |
| Integration Test | Small File Sync | 4. Transfer Test: Successfully transfer a small, non-audio text file from the ESP32 (reading from the SD card) to a debugging tool (like a BLE scanner app on a phone), proving the chunking logic works. |

Use this video to understand the hardware integration

https://youtu.be/RvbWl8rZOoQ?si=9w5sti54g-XXrRNz

TIMELINE

| Week | Integrator (You) | P1 (App Dev - AI) | P2 (App Dev - UI) | P3 (Hardware - Firmware) | P4 (Hardware - BLE) | P5 (Testing & Docs) |
| --- | --- | --- | --- | --- | --- | --- |
| W1: Foundation | Support parts ordering & setup. Finalize all technical specs (Pinout, UUIDs, data format). | Setup Python Env. Research S-to-T/Summarizer APIs. Build AI Script 1 (Manual audio $\rightarrow$ Raw Text). | Choose App Platform (Flutter/App Inventor). Design UI wireframes. Build App Shell (empty screens). | Order Parts. Set up Arduino IDE (C++). Write I2S Audio Capture firmware. | Order Parts. Implement basic BLE GATT Server advertising ("EduNote Watch"). | Define 10 Test Scenarios & Success Metrics. Create report templates. |
| W2: Connectivity | Coordinate P4/P2 Interface (BLE handshake protocol). Start end-to-end data flow plan. | Refine AI Script 1. Add basic text cleaning (filler word removal). Set up a simple cloud/local API endpoint for the app to call. | Implement BLE Client on the mobile app side (Android is simplest). Build the Connect/Pairing UI. | SD Card File Management: Implement continuous writing to SD card. Add WAV Header Finalization logic (Crucial Step). | Implement Chunking Logic: Successfully read a small dummy file from SD card and transmit it reliably to a mobile app debugger. | Run Unit Tests on P3's I2S capture quality and P4's basic BLE connection reliability. |
| W3: Integration | CRITICAL: Oversee full Watch $\rightarrow$ App $\rightarrow$ AI flow. Measure transfer speed and latency. | Integrate AI Script 1 with App: P2 sends audio file to P1's API endpoint, P1 returns raw transcribed text. | Implement File Reception: P2 reassembles the BLE audio chunks into a valid .wav file on the phone. Display Raw Transcript returned from P1. | Work with P4 to integrate the SD Card Read Function into the BLE transfer service. | Finalize File Transfer Service: Achieve reliable transfer of the full 10-minute WAV file from the watch to the mobile app. | Run Integration Test 1 (End-to-end flow). Report on failure points in transfer/transcription. |
| W4: Verification | Review AI Output Quality. Coordinate feedback loop between P1, P5, and P2 on note structure. | Implement Confidence Checker: Integrate Wikipedia API/GPT cross-checks into the pipeline. Refine Summarization: Optimize T5/Pegasus prompt for structured academic notes. | Finalize Notes Display: Implement rendering of structured notes, and visually show Confidence Flags returned by P1. | Focus on Low-Power Mode and final power consumption optimization. | Implement Error Handling for BLE (timeouts, disconnects, re-tries) for robustness. | Run User Feedback Session with test group. Document all needed modifications to the notes format and UI. |
| W5: Debug & Stress | Lead daily Bug Triage Meetings. Prioritize fixes based on P5's stress reports. | Debug and Optimize the AI pipeline for speed and accuracy (especially accent tuning). Finalize clean, documented AI code. | Implement Export Functionality (PDF/DOCX mock-up). Fix all UI/UX bugs reported by P5. | Stress Testing: Run continuous recording tests (longer than 10 minutes) and battery life tests. | Final Debugging of connection stability. Prepare Watch/Firmware for final flashing/installation. | Lead Stress Testing (e.g., 2-hour recording). Run Regression Testing after every bug fix. Finalize detailed test report. |
| W6: Finalization | Prepare Final Presentation and demo script. Ensure the demo is flawless and repeatable. | Prepare Code Documentation and summary of AI choices for the final report. | Finalize app look and feel. Create video/slides of the app interface for the presentation. | Final Flashing and assembly of the prototype into the casing. | Final Check of all BLE UUIDs and communication protocols for the report. | Final Report writing, including all test results and future scope. Practice the presentation. |

Tasks assigned (WEEK 1)

| Role | Responsibility | Key Deliverables (End of Week 1) | Detailed Steps & Explanation |
| --- | --- | --- | --- |
| P6: PRIYA(SOFTWARE AND HARDWARE INTEGRATION) | Support & Scoping | 1. All Parts Ordered (P3/P4). 2. Final Pinout List. 3. Test Audio Data Set. | Support P3/P4 (Hardware): Review their parts list for compatibility (e.g., ensure the MEMS mic is I2S). Confirm the final GPIO pins for the microphone and SD card (the Pinout List). Support P1/P2 (App/AI): Source 3-5 sample 10-minute lecture audio files (with various accents/noise levels) to create the Test Data Set. This allows the AI team to work immediately. |
| P1:MONISHA (APP DEVOLOPEMENT) | AI Environment Setup & Research | 1. Python Environment Ready (Virtual Env). 2. S-to-T/Summarizer Model Choices. 3. Basic "Hello World" Python Script. | Setup: Install Python, create a Virtual Environment (venv), and install core libraries (requests, pydub, openai/transformers). Research: Choose specific versions/APIs for Whisper (or a local variant) and T5/Pegasus for summarization. The "Hello World" script should just confirm the API keys/models are working (e.g., submit a single sentence to a summarizer and print the result). |
| P2: GREESHMITA (APP DEVOLOPEMENT) | UI Research & Design | 1. App Platform Chosen (Flutter/App Inventor). 2. UI Sketch/Wireframe (3 Screens). 3. Basic "View Notes" Mockup Screen. | Design: Sketch the three critical screens: Connect Watch (scan/pair/sync button), Sync Progress (file transfer bar), and View Notes (where the final notes display). Tool Choice: Select the app development environment (Flutter is recommended for cross-platform, but App Inventor is faster for MVP). Start building the empty shell of the View Notes Screen. |
| P3: SAI HARSHITHA (HARDWARE) | Part Ordering & I2S Setup (C++) | 1. ALL Parts Ordered. 2. ESP32 IDE Setup (C++). 3. I2S Config Code Done. | Ordering: Finalize the order list (ESP32-S3, I2S Mic, SD Module, Battery). Setup: Install the Arduino IDE and the ESP32 board package. Code: Adapt the C++ code I provided to define the specific GPIO pins and write the I2S Configuration code (setting the sample rate, pins, and master/receive mode). This is the digital ear of the watch. |
| P4: SHWETHA (BLUETOOTH) | Part Ordering & BLE Research (C++) | 1. ALL Parts Ordered. 2. BLE GATT Research Complete. 3. Basic BLE Server Running. | Research: Understand the Bluetooth Low Energy (BLE) concepts: GATT, Services, and Characteristics. Your goal is to use the ESP32 as a BLE Server. Code: Get the standard ESP32 BLE example code running, successfully advertising as "EduNote Watch" and printing a simple "Hello" notification to a phone app (like nRF Connect). |
| P5: HARSHITHA K M (TESTING) | Test Case Creation & Templates | 1. Test Scenarios (x10) Defined. 2. Bug Report/Progress Report Template. 3. Defined Success Metrics. | Testing Strategy: Create a list of 10 specific, realistic test scenarios (e.g., "Teacher whispers definition," "Loud fan noise," "Teacher speaks for 10 min non-stop"). Define the Success Metrics: (e.g., Transcription accuracy > 90%, Verification flags < 5% of sentences). Prepare simple templates for logging bugs and reporting weekly progress. |

Items to order

| Component | Recommended Model/Specification | Quantity (1 Unit) | Key Technical Specification | Cost Range (INR, Approx.) |
| --- | --- | --- | --- | --- |
| 1. Microcontroller (MCU) | ESP32-S3-DevKitC-1 (or similar S3 module) | 1 | Processor: Xtensa LX7 Dual-Core. Wireless: Wi-Fi + BLE 5.0 (Critical for App Sync). Interfaces: Dedicated I2S, SPI, I2C. | ₹800 – ₹1000 |
| 2. Digital Microphone | INMP441 MEMS I2S Module | 1 | Output: Digital I2S. SNR: 61 dBA (High quality). Required I2S Configuration: 16 kHz Sample Rate, 16-bit Depth, Mono Channel. | ₹170 – ₹350 |
| 3. Local Storage Module | MicroSD Card Module (with voltage regulator) | 1 | Interface: SPI (High-speed bus for file writing). Voltage: 3.3V/5V compatible. | ₹60 – ₹150 |
| 4. Storage Media | MicroSD Card (8GB or 16GB) | 1 | Speed: Minimum Class 10 (Essential for continuous high-speed audio file writing). | ₹200 – ₹400 |
| 5. Display | 0.96 inch OLED Display (Monochrome/Blue) | 1 | Driver: SSD1306. Interface: I2C (2-wire for simple setup). Resolution: 128x64 pixels. | ₹100 – ₹200 |
| 6. Battery | 3.7V Li-Po Battery | 1 | Capacity: 800mAh to 1000mAh (Aim for higher capacity for longer recording life). Includes PCM (Protection Circuit Module). | ₹300 – ₹550 |
| 7. Charger | TP4056 Charger Module (Micro/Type-C USB) | 1 | Function: Safe charging and protection for the 3.7V Li-Po battery (prevents overcharge/over-discharge). | ₹20 – ₹50 |
| SUBTOTAL (Approx. INR) |  |  |  | ₹1,650 – ₹2,750 |

Lab & Commodity Items

| Component | Specification | Rationale | Compatibility & Usage |
| --- | --- | --- | --- |
| Jumper Wires | Mixed male-male & male-female (20cm) | Essential for connecting the modules to the ESP32-S3 development board headers during prototyping. | Used for all GPIO and interface connections (I2S, SPI, I2C). |
| Tactile Push Buttons | 6x6mm momentary buttons | At least one is needed for the core "Start/Stop Recording" function. | Connects directly to ESP32 GPIO pins with minimal external components (often just a pull-up/down resistor). |
| LEDs & Resistors | Standard 5mm LEDs (Red, Green, or Blue) + 220 Ohm Resistors | Used as visual indicators for system status (e.g., Green = Recording, Blue = BLE Connected). | Resistors are crucial for protecting the LEDs when powered by the 3.3V/5V lines. |
| 3D Printing Material | PLA/PETG Filament | Needed for designing the custom, lightweight wearable casing (watch enclosure). | Required by P5/Integrator to build the final prototype housing. |
| Basic Tools | Soldering Iron, Solder, Multimeter, Wire Stripper | Necessary assembly tools. | Should be available in the college lab; avoid buying unless necessary. |

These are the ONLY things that will break Week-1:

🚫 Blocker 1 — I2S audio must stream to Serial Monitor

(If not → Week-2 collapses)

🚫 Blocker 2 — BLE advertisement must appear in nRF Connect

(If not → no file transfer testing)

🚫 Blocker 3 — Audio parameters MUST match

(16kHz, 16-bit PCM, mono)

🚫 Blocker 4 — Android Native must be chosen this week

(Flutter BLE is too slow)

🚫 Blocker 5 — Git repository must be created

(No shared repo = chaos later)

⚠️ 5. Risk-Management Plan — FINAL VERSION

Risk A: BLE Transfer too slow

Mitigation:

- Accept 12–30 mins transfer time

- Consider compressing audio later

Risk B: I2S microphone noisy / distorted

Mitigation:

- Test multiple gain settings

- Lower sample rate to reduce data size/noise

- Shield microphone from fan/wind

Risk C: BLE disconnects mid-transfer

Mitigation:

- Implement retry for each chunk

- At minimum: confirm chunk number

Risk D: AI fails due to accent

Mitigation:

- Whisper medium or Whisper-large

- Try with noise-reduced sample audio

Risk E: App team gets stuck

Mitigation:

- Build UI with dummy data first

- Integration later

Risk F: Git conflicts

Mitigation:

- Use main, dev, and feature branches

- Only integrator merges into dev/main